Projeto I

EST

Group 6:

Fábio Magalhães – A75030

Rui Carvalho – A76279

Index

Problem Statement	3

Market Study	3

Constraints	4

Technical Constraints	4

Functional Requirements	4

Non-Functional Requirements	4

Hardware Specifications	5

Software Specifications	5

System Overview	6

Events	6

Use Cases	7

State Chart	7

Sequence Diagram	8

System Stack	8

Gantt Chart	9

Problem Statement

In today society mankind is constantly using digital devices, that are affecting, negatively, vision health. It's been a concern for the past years to fight this era’s side-effect. To get yourself diagnosed, you must go to a doctor, which in public healthcare means a long wait and sometimes may be costly.

The Project’s goal is to design a tool to help diagnose irregularities in human vision, such as astigmatism, near-sightedness, and colour blindness. With only one device the user will be able to self-diagnose in an interactive and fast way.

We will have a screen showing images, that the user must identify. The device will recognise the answer and evaluate it. In the end of the test it will give you a result of how good or not your vision is.

Market Study

There are already a lot of vision diagnostics devices. Although these devices are all equipment that should be handle by a professional technician, we found some that compare to our project.

SPOT Vision Screener:

Professional, portable device. Unlike ours, the device diagnoses the patient through a camera, screening the eye directly. This equipment is extremely expensive.

Medivision Digital Acuity Chart:

Similar to our project, it displays the test in a screen. Although the diagnostic must be made by a doctor/technician, as the device will not give any. The device, though cheaper than SPOT, is still very expensive.

Resuming there exists already good devices that perform the task, but they are very expensive and they all need an experienced technician to use them.

Constraints

- Budget must be minimal.

- Project developed by a team of two.

- Project must meet the final deadline.

Technical Constraints

- STM32F4-DISCOVERY board

- Keil uVision MDK-ARM

- Digital Signal Processor (DSP)

- FreeRTOS

- Minimum of three sensors

Functional Requirements

- Diagnose User

- Display the vision test on the TFT screen.

- Detect user interaction.

- Measure the distance between the device and the patient.

- Check suitability of the ambient light.

- Recognise speech input data.

- Show malfunctions on the display.

- Give audio signals.

Non-Functional Requirements

- Low Cost and low power.

- User-friendly interface.

- Small and portable.

- The system must have low latency.

Hardware Specifications

- STM32f4 Discovery Board

  - STM32F407VGT6 MCU

  - 32-bit ARM Cortex-M4 CPU with FPU core

  - 1-Mbyte Flash Memory

  - 192-Kbyte RAM

  - MP45DT02 ST MEMS Digital Microphone

  - CS43L22 Audio DAC

  - Up to 17 timers

  - 3 ADC 12-bit (up to 24 channels)

  - General-Purpose DMA

- Light sensor

  - LDR 1K-10K Ω

- Ultrasonic distance measurement

  - HC-SR04

  - 2-400 cm of non-contact measurement

- MEMS Digital Microphone onboard

- TFT w/ Touchscreen

  - ILI9488 Display Driver

  - 320x480 Resolution

  - Resistive Touchscreen

STM32F4 Discovery Board

The STM32F4DISCOVERY Discovery allows development with the STM32F407VG high performance microcontroller with the ARM® Cortex®-M4 32-bit core.

Based on STM32F407VG, it includes an ST-LINK/V2 embedded debug tool, two ST-MEMS digital accelerometers, a digital microphone, one audio DAC with integrated class D speaker driver, LEDs, push buttons and a USB OTG micro-AB connector.

LDR

Specifications

| Bright Resistance (100 lux) | 5k Ω |
| --- | --- |
| Dark Resistance (10 lux) | 10k Ω |
| Very Bright Resistance (100 lux) | 400 Ω |
| Max Voltage | 150 V |
| Max Power | 100 mW |
| Ambient Temperature Range | -30 ~ +70 °C |

Interface

|  | Pin Name | Pin Function |
| --- | --- | --- |
| LDR OUT | PC5 | ADC12_IN15 |

Test Cases

Vo=VCC-VCCLDRLDR+R			Vo=5-5LDRLDR+1K

Best ambient light (250 lux) -> 2.5k Ω

| Light Level | Expected Output | Real output |
| --- | --- | --- |
| Bright (100 lux) | 0.83 V |  |
| Dark (10 lux) | 0.45 V |  |
| Good Lighting (250 lux) | 1.43 V |  |

HC-SR04

Specifications

| Working Voltage | 5V |
| --- | --- |
| Working Current | 15mA |
| Working Frequency | 40KHZ |
| Max Range | 4m |
| Min Range | 2cm |
| Measuring Angle | 15° |
| Trigger Input SIgnal | 10us TTL pulse |

Interface

|  | Pin Name | Pin Function |
| --- | --- | --- |
| Trig | PA1 | Out |
| Echo | PA0 | TIM2_CH2 |

Test Cases

Dist=HighLevelTime×SoundSpeed2

D=Thi×3402

| Distance | Expected Output | Real output |
| --- | --- | --- |
| Min (2 cm) | 2 cm |  |
| Max (400 cm) | 400 cm |  |
| Reference (150 cm) | 150 cm |  |

MEMS Microphone

Specifications

| Parameter | Min | Typical | Max |
| --- | --- | --- | --- |
| Supply voltage | 1.6V | 1.8V | 3.6V |
| Current in normal mode |  | 0.65mA |  |
| Current in power-down mode |  | 0.20mA |  |
| Sensitivity | -29dBFS | -26dBFS | -23dBFS |
| Input clock frequency | 1 MHz | 2.4MHz | 3.25 MHz |
|  |  |  | 10ms |
| Low level logic input/output voltage | -0.3 V |  | 0.35*Vdd |
| High level logic input/output voltage | 0.65*Vdd |  | Vdd+0.3 |

Interface

|  | Pin Name | Pin Function |
| --- | --- | --- |
| PDM Out | PC3 |  |
| Clock | PB10 | TIM2_CH3 |

Test Cases

|  | Expected Output | Real output |
| --- | --- | --- |
| Speak to microphone | Voice input |  |

Display

ILI9488 Display Driver

SPI Compatible

Specifications

| Resolution | 320x480 |
| --- | --- |
| Frame Memory | 345 kbytes |
| Input Image | 24 bits |
| IO VCC | 1.65 – 3.3 V |
| Temperature | -40 ~ +85 °C |

Interface

|  | Pin Name | Pin Function |
| --- | --- | --- |
| SCK | PA5 | SPI1_SCK |
| MISO | PA6 | SPI1_MISO |
| MOSI | PA7 | SPI1_MOSI |

Test Cases

| Send Command | Expected Output | Real output |
| --- | --- | --- |
| 0x01 | Module Resets |  |
| 0x04 | Manufacturer, Version, ID |  |

Resistive Touchscreen

Specifications

| Size | 74*49 mm |
| --- | --- |

Interface

|  | Pin Name | Pin Function |
| --- | --- | --- |
| Y+ | PB0 | ADC12_IN8, GPIO IN/OUT |
| Y- | PB1 | ADC12_IN9, GPIO IN/OUT |
| X+ | PE7 | GPIO IN/OUT |
| X- | PE8 | GPIO IN/OUT |

Test Cases

| Touch | Expected Output (X,Y) | Real output (X,Y) |
| --- | --- | --- |
| Right Top Corner | 0,49 mm |  |
| Right Bottom Corner | 0,0 mm |  |
| Left Top Corner | 74,49 mm |  |
| Left Bottom Corner | 74,0 mm |  |

Software Specifications

- Keil uVison MDK-V5

- FreeRTOS

- C Language

- MatLab

System Overview

Events

| Event | System Response | Source |
| --- | --- | --- |
| On/Off | Turn System On/Off | User |
| Read Light | Reads ADC and processes data | Local Sys |
| Read Distance | Gets distance value | Local Sys |
| Display | Changes display data | Local Sys |
| Get voice | Gets and recognises voice input | User |
| Get Touch | Gets user input on touchscreen | User |

Use Cases

State Chart

Sequence Diagram

System Stack

Task Priority

Gantt Chart